OPPO A32/A53/A33 Service Manual -V1.0

Confidential

Content

1. Guidance	3

1.1 Warnings	3

1.2 Cautions	3

2. General Repair Information	4

3. Phone Structure Diagram	5

3.1 Product Appearance	5

3.2 Phone Structure Explosive View	5

3.3 Explosive Map BOM	6

4. Disassembly Steps	9

4.1 Disassembly Tools	9

4.2 Preparation	9

4.3 Disassembly Steps	9

5. Assembly Steps	13

5.1 Bill of Must-Be-Replaced Materials	13

5.2 Assembly Steps	13

6. After-sale calibration	16

6.1 Leak Calibration	16

6.1.1 Calibration Scenario	16

6.1.2 Calibration Tool	16

6.1.3 Calibration Method	16

6.2 Fingerprint optical calibration	17

6.2.1 Calibration Scenario	17

6.2.2 Calibration Tool	17

6.2.3 Calibration Method	17

6.3 Dual Camera Calibration	17

6.3.1 Calibration Scenario	17

6.3.2 Calibration Tool	17

6.3.3 Calibration Method	17

7.  Function Test	19

7.1 Test Path	20

7.2 Test Requirements	20

8. General Cautions	22

- Guidance

The purpose of this document is only to guide OPPO technicians to carry out maintenance service to OPPO products. The content shall be keep in confidential, only avail be able to OPPO authorized service centers, and cannot be provided to third parties without authorization. Please follow the regulations and the guidance for maintenance service. For any problems, please contact China HQ in time.

1.1 Warnings

1. Care must be taken on installation in vehicles fitted with electronic engine management systems and ABS (Anti–skid Braking Systems). Under certain fault conditions, emitted RF energy can affect their operation. If necessary, consult the vehicle dealer/manufacturer to determine the immunity of vehicle electronic systems to RF energy.

2. Phones must not be operated in areas likely to contain potentially explosive atmospheres, e.g. petrol stations (service stations), blasting areas etc.

3. Operation of any radio transmitting equipment, including cellular telephones, may interfere with the functionality of inadequately protected medical devices. Consult a physician or the manufacturer of the medical device if you have any questions. Other electronic equipment may also be subject to interference

1.2 Cautions

1. Repair services and calibrations must be undertaken by qualified personnel only.

2. Use only approved components as specified in the parts list.

3. Ensure all components, modules screws and insulators are correctly re-fitted after servicing and alignment.

4. Electrostatic discharge is the main cause of damage to sensitive components of electronic products. Service centers must operate in accordance with OPPO's requirements for ESD protection. Ensure all work is carried out at an ESD workstation and that an ESD wristband is worn.

- General Repair Information

- Ensure all work is carried out at an ESD workstation and that an ESD wristband is worn before starting maintenance.

- Wear ESD gloves to avoid oil stains and fingerprints.

- Use protective films to protect the display, camera, camera lens to prevent dust and scratches.

- Use a clean cloth, ESD brush and alcohol (Concentration above 95%) for appearance cleaning. Do not use other items (such as an eraser) for cleaning to prevent the protective layer from being scratched, which may lead to oxidation and corrosion.

- Faulty welded mechanical parts (except shield cover and shield frame parts) can only be replaced and not repairable.

- Use accessories provided by OPPO.

- Check the contacts or solder joints of devices that may cause simple faults (e.g. soldered interfaces or switches). Re-weld if necessary (only for service centers where lead-free soldering is available) and clean the flux remaining after soldering.

- Use the equipment provided by OPPO to test the phone (e.g. When the charging function of the device is abnormal, do use the original OPPO adapter to test to ensure the accuracy of the test results).

- Fill in the fault phenomenon code and fault reason code according to the actual situation, and accurately enter the replacement material code in the CRM system.

- For more information about products,

Mainland China please visit:

http://172.16.41.122:8888/kbaseui-dev/main.do

Regions except for Mainland China please visit:

ftp://ftpex.oppo.com:919/ or

https://oppo knowledge.custhelp.com/app/home/loc/zh_CN

3. Phone Structure Diagram

3.1 Product Appearance

As shown in Figure 3-1-1

3.2 Phone Structure Explosive View

As shown in Figure 3-2-1

Figure 3-2-1

3.3 Explosive Map BOM

| NO | Material Name | Physical Figure | Sub material | QTY | Unit |
| --- | --- | --- | --- | --- | --- |
| 1 | Screen |  | （1）OLED Display Screen | 1 | PCS |
|  |  |  | （2）Middle Frame | 1 | PCS |
|  |  |  | （5）receiver | 1 | PCS |
|  |  |  | （17）motor | 1 | PCS |
|  |  |  | FPC VFA218-0- Patch 20021 | 1 | PCS |
|  |  |  | FPC PFA218-0- Patch 20021 | 1 | PCS |
| 2 | Mainboard |  | （4）2FA218-SB 20221(128GB 6GB) | 1 | PCS |
| 3 | Back cover |  | （11）Back cover | 1 | PCS |
| 4 | Rear main |  | （6）OV13B10 13M 8.5*8.5*4.74 5P BTB LED321AM Huaqin | 1 | PCS |
| 5 | Rear depth camera |  | （6）GC02M1B 2M 6.0*5.7*3.15 3P BTB | 1 | PCS |
| 6 | Rear macro camera |  | （6）GC02K0 2M 6*5.7*3.17 3P BTB VC22968 Huaqin | 1 | PCS |
| 7 | Front camera |  | （3）OV8856 8M 7.35×6.3×4.49 5P BTB AM809B | 1 | PCS |
| 8 | Small board |  | （16）PCB AFA218-SB 20222 | 1 | PCS |
| 9 | Motherboard lower cover |  | （7）Motherboard lower cover | 1 | PCS |
| 10 | battery |  | （13）battery | 1 | PCS |
| 11 | Fingerprint components |  | （12）Fingerprint components | 1 | PCS |
| 12 | Speaker BOX |  | （18）speaker BOX | 1 | PCS |
| 13 | Camera lens assembly |  | （8）Camera decoration ring | 1 | PS |
|  |  |  | Camera lens | 1 | PCS |
| 14 | C board |  | (15)FPC CFA218-SA 20021 | 1 | PCS |
| 15 | RF line |  | （14）RF Connection line | 1 | PCS |
| 16 | SIM card tray |  | （9）SIM card tray | 1 | PCS |
| 17 | Screw |  | （10）Screw | 1 | PCS |

Note：This table is used only for the name of the annotation but not for the preparation of materials. Please refer to the Price List of OPPO Mobile Phone Original Accessories (for Mainland China)/Price List of Commonly-Used Materials and Accessories (Regions except for Mainland China) for the preparation.

4. Disassembly Steps

4.1 Disassembly Tools

4.2 Preparation

1. The phone must be powered off before disassembly.

2. Way to Power Off:

System Power Off: Press the power key then slide to power off.

Forced Power Off: Long press the volume “+” key and power key for about 8 seconds until the display is off.

4.3 Disassembly Steps

| Step | Content | Figure | Cautions |
| --- | --- | --- | --- |
| 1 | Remove the SIM tray |  | Don't scratch the device |
| 2 | Remove the back cover |  | Avoid heating the camera. Heating requirements: 70 ° C -80 ° C for 2-4 mins; (2) Control the strength when the battery cover and the middle frame are separated. |
| 3 | Clean Double-sided tape |  | Please confirm whether the rear decorative ring is degummed. If it is loose, replace the back glue of the decorative part and press it hard. |
| 4 | Remove the Screws and Motherboard lower cover/camera decoration ring |  | 1. Make sure to remove the screws before removing the motherboard bracket, otherwise the motherboard bracket will be broken easily 2. After removing the camera decoration ring, pay attention to clean up the residual glue |
| 5 | Remove fingerprint module and camera |  | 1 Disassemble the fingerprint module and be careful not to fold the fingerprint FPC 2. The damaged camera conductive cloth needs to be replaced with new materials |
| 6 | Remove the battery |  | Add 0.2ml of alcohol to the top and bottom of the handle, and stand the phone sideways for 15 seconds until the alcohol spreads; Note: During disassembly of the battery, it is necessary to avoid bending and deformation of the middle part of the battery as much as possible. |
| 7 | Remove the motherboard |  | 1. Remove the BTB button on the motherboard before removing the motherboard 2. Pay attention to clean the thermal gel after removing the motherboard |
| 8 | Remove the receiver/speaker/small board/motor/RF cable/side button |  | / |

Figure 4.3 All Materials of the Phone

5. Assembly Steps

5.1 Bill of Must-Be-Replaced Materials

| No. | Code | Material | SPEC. | QTY | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | 4883761 | Battery pull tape | BLP803 transparent | 1 | Remove the battery must be replaced |
| 2 | 4882732 | Battery cover double-sided tape | FA217 | 1 | Must be replaced after disassembly |

5.2 Assembly Steps

| Step | Content | Figure | Cautions |
| --- | --- | --- | --- |
| 1 | Install side key FPC |  |  |
| 2 | Install receiver/small board/motor |  | Before installing the receiver, confirm whether the soft pad of the receiver is damaged, if it is damaged, replace it with a new one |
| 3 | Install the motherboard and buckle the RF cable |  | Pay attention to applying thermal gel before installing the motherboard |
| 4 | Install C board/install speaker |  | / |
| 5 | Install the camera |  | 1. The damaged conductive cloth needs to be replaced with new materials 2. Be careful not to break the BTB button |
| 6 | Install the battery |  | Battery pull tape needs to be replaced |
| 7 | Install fingerprint module |  | / |
| 8 | Install the camera decoration ring/install the lower cover/screw |  | 1. The double-sided tape of the camera decoration ring needs to be replaced with new materials |
| 9 | Install the battery cover |  | The Battery cover double-sided tape needs to be replaced with new materials |

6. Calibration

After replacing the camera, it must be calibrated

| Enter Password: 6776 | Click the photo button | Check "Confirm" after calibration |
| --- | --- | --- |

7.  Function Test

7.1 Test Path

Dialing Interface>Enter*#899#>Enter automatic test； The "Confirm" dialog box will pop up after the test complete, if there is no problem choose “Pass”, otherwise choose “Fail”.

7.2 Test Requirements

| No. | Test Items | Test Requirements |
| --- | --- | --- |
| 1 | LCD Test | Observe the red, green, blue, white, black, gray, grayscale and color interface successively; If there are black spots, bright spots and other anomalies, the function is good; |
| 2 | Vibration Test | Get ear close to check the vibration from the motor for 3 seconds. If the motor vibrates normally(No noises or weak vibration), the function is good; |
| 3 | Touch Screen Auto Test | If the interface shows “PASS” and jumps to the next test, the function is good; |
| 4 | Front Camera Preview | Align the camera with a white surface and a black surface. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, and there is no bright spot on the screen after aligning the black surface, the camera is good; |
| 5 | Rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is good; |
| 6 | Second rear camera preview | Align the Second rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is good; |
| 7 | third rear camera preview | Align the third rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is good; |
| 8 | Echo test | Testing Main MIC: Blow the air to the MIC hole on the bottom of the phone. If the receiver (earphone) sounds, the function of Main MIC is good; Testing Sub MIC: Blow the air to the sub MIC hole on the top of the phone. If the speaker sounds, the function of Main MIC is good; |
| 9 | Flashlight Test | Open the flashlight. If it can be opened normally with color deviations, the function is good; |
| 10 | Charging Test | Use the original OPPO adapter and USB cable to charge. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 11 | Headset in-out Plug test | Plug in and out the earphone with the OPPO earphone. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 12 | Sensor Self-test and Calibration | When the phone enters into the Sensor self-calibration interface, the phone must be placed on the desk flatly. Then click the first calibration item. After the test is passed, it will automatically jump to the next test item; |
| 13 | M-Sensor Test | Shake the phone from the left to the right side. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 14 | Proximity-sensor Test | Use the palm to cover the light sensor hole, and the screen will turn green from black. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 15 | Keypad Test | Press the power key and volume key one by one. If keys can be pressed normally and the interface jumps to the next test, the function is good; |
| 16 | Media test | Click the “20-4K（-3db)signal” to test the sound of the media function for 5s. If there is no silence, weak sound, noises, and breaking sound, the interface shows “PASS” and jumps to the next test, the function is good; |
| 17 | Call test | If the phone has been installed with the operator's SIM card, dial the corresponding operator's phone number. If no card is installed, call the 112 to test (Calling the Operator is recommended). If the call can be performed normally and the receiver is without noises or weak sounds, the function is good; |

8. General Cautions

1. When disassembling (front/rear) camera pay attention to protect the camera lens with low mucosa;

2. If it involves replacing the screen assembly, the main board or cleaning the original thermal gel, it is necessary to apply tShe thermal conductive gel according to the above installation steps;

3. Please confirm whether the rear decorative ring is degummed. If it is loose, replace the back glue of the decorative part and press it hard.

For OPPO A53/A33 Disassembly Video Please visit:

FTP: Overseas Customer Service--- OPPO Technical Service--- Product Material---OPPO A53 (CPH2127).

| Version | Date | Draft | Review | Approve | Modified content |
| --- | --- | --- | --- | --- | --- |
| V1.0 | 2020-08-24 | Lyndon | Lin binbin | Huang guoxin | 1.First Edition |