Reno4 Pro 5G Service Manual _V1.0

Confidential

Content

1.	Guidance	3

1.1 Warnings	3

1.2 Cautions	3

2.	General Repair Information	4

3. Phone Structure Diagram	5

3.1 Product Appearance	5

3.2 Phone Structure Explosive View	5

3.3 Explosive Map BOM (The number in brackets is the Figure 3-2-1 number)	6

4. Disassembly Steps	12

4.1 Disassembly Tools	12

4.2 Preparation	12

4.3 Disassembly Steps	12

5. Assembly Steps	20

5.1 Bill of Must-Be-Replaced Materials	20

5.2 Assembly Steps	20

6. Calibration	27

6.1 AfterSale Devices Calibrate	27

6.1.1 Leak Calibration	27

6.1.2 Finger print optical calibration	28

6.1.3 TP calibration	29

6.1.4 Aftersale Triple camera calibration	30

6.1.5 OIS-Still	31

6.1.6 Smart power amplifier calibration	31

7.  Function Test	33

7.1 Test Path	33

7.2 Test Requirements	33

8. General Cautions	37

- Guidance

The purpose of this document is only to guide OPPO technicians to carry out maintenance service to OPPO products. The content shall be keep in confidential, only avail be able to OPPO authorized service centers, and cannot be provided to third parties without authorization. Please follow the regulations and the guidance for maintenance service. For any problems, please contact China HQ in time.

1.1 Warnings

- Care must be taken on installation in vehicles fitted with electronic engine management systems and ABS (Anti–skid Braking Systems). Under certain fault conditions, emitted RF energy can affect their operation. If necessary, consult the vehicle dealer/manufacturer to determine the immunity of vehicle electronic systems to RF energy.

- Phones must not be operated in areas likely to contain potentially explosive atmospheres, e.g. petrol stations (service stations), blasting areas etc.

- Operation of any radio transmitting equipment, including cellular telephones, may interfere with the functionality of inadequately protected medical devices. Consult a physician or the manufacturer of the medical device if you have any questions. Other electronic equipment may also be subject to interference

1.2 Cautions

- Repair services and calibrations must be undertaken by qualified personnel only.

- Use only approved components as specified in the parts list.

- Ensure all components, modules screws and insulators are correctly re-fitted after servicing and alignment.

- Electrostatic discharge is the main cause of damage to sensitive components of electronic products. Service centers must operate in accordance with OPPO's requirements for ESD protection. Ensure all work is carried out at an ESD workstation and that an ESD wristband is worn.

- General Repair Information

- Ensure all work is carried out at an ESD workstation and that an ESD wristband is worn before starting maintenance.

- Wear ESD gloves to avoid oil stains and fingerprints.

- Use protective films to protect the display, camera, camera lens to prevent dust and scratches.

- Use a clean cloth, ESD brush and alcohol (Concentration above 95%) for appearance cleaning. Do not use other items (such as an eraser) for cleaning to prevent the protective layer from being scratched, which may lead to oxidation and corrosion.

- Faulty welded mechanical parts (except shield cover and shield frame parts) can only be replaced and not repairable.

- Use accessories provided by OPPO.

- Check the contacts or solder joints of devices that may cause simple faults (e.g. soldered interfaces or switches). Re-weld if necessary (only for service centers where lead-free soldering is available) and clean the flux remaining after soldering.

- Use the equipment provided by OPPO to test the phone (e.g. When the charging function of the device is abnormal, do use the original OPPO adapter to test to ensure the accuracy of the test results).

- Fill in the fault phenomenon code and fault reason code according to the actual situation, and accurately enter the replacement material code in the CRM system.

- For more information about products,

Mainland China please visit:

http://172.16.41.122:8888/kbaseui-dev/main.do

Regions except for Mainland China please visit:

ftp://ftpex.oppo.com:919/ or

https://oppo knowledge.custhelp.com/app/home/loc/zh_CN

3. Phone Structure Diagram

3.1 Product Appearance

As shown in Figure 3-1-1

Figure 3-1-1

3.2 Phone Structure Explosive View

As shown in Figure 3-2-1

Figure 3-2-1

3.3 Explosive Map BOM (The number in brackets is the Figure 3-2-1 number)

| No. | Material Name (After-sales) | Physical Figure | Specifications | QTY |  | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Rear camera lens |  | (1 for Main & Telephoto camera lens) Rear camera lens 3 | 2 |  | PCS |  |
|  |  |  | （1 for Wide Angle Camera） Rear camera lens | 1 |  | PCS |  |
| 2 | Back cover |  | (1)Rear camera lens | 3 |  | PCS |  |
|  |  |  | (2)Back cover | 1 |  | PCS |  |
| 3 | (3)Rear camera （Can be disassembled separately for repair） |  | （top/ 48M Main Camera）Camera | 1 |  | PCS |  |
|  |  |  | （middle/12M Wide Angle Camera）Camera | 1 |  | PCS |  |
|  |  |  | （lower/ 13M Telephoto camera）Camera | 1 |  | PCS |  |
|  |  |  | camera combination copper foil sheet(Rear) | 1 |  | PCS |  |
|  |  |  | Rear camera bracket | 1 |  | PCS |  |
| 4 | (4)Mainboard bracket |  | Mainboard bracket | 1 |  | PCS |  |
|  |  |  | FPC patch semi-finished product（Flash-FPC） | 1 |  | PCS |  |
| 5 | (5)Mainboard |  | PCB semi-finished（2A-PCB） | 1 |  | PCS |  |
|  |  |  | Light sensor silicone sleeve | 1 |  | PCS |  |
|  |  |  | Laser sensor Silicone rubber Case | 1 |  | PCS |  |
|  |  |  | REC conductive copper foil | 1 |  | PCS |  |
| 6 | （6）FPC patch semi-finished product |  | UAE008-0-patch 19191（USB-FPC） | 1 |  | PCS |  |
| 7 | （7）FPC patch semi-finished product |  | LAE008-0-patch 19191（main-FPC） | 1 |  | PCS |  |
| 8 | （8）Receiver |  | Top Speaker | 1 |  | PCS |  |
|  |  |  | Receiver sealing sponge adhesive | 1 |  | PCS |  |
|  |  |  | Receiver FPC double-sided adhesive | 1 |  | PCS |  |
|  |  |  | REC conductive copper foil | 1 |  | PCS |  |
| 9 | （9）Front camera |  | camera（32M Front camera） |  | 1 |  | PCS |  |
|  |  |  | camera conductive copper foil(Front) |  | 1 |  | PCS |  |
|  |  |  | Front camera grounded foam | 1 |  | PCS |  |
|  |  |  | Front camera fixed double-sided adhesive | 1 |  | PCS |  |
| 10 | Screen |  | (10)OLED screen | 1 |  | PCS |  |
|  |  |  | (11)Touch Panel Assembly | 1 |  | PCS |  |
|  |  |  | (12)Motherboard upper cover | 1 |  | PCS |  |
| 11 | （13）SIM bracket |  | （13）SIM bracket | 1 |  | PCS |  |
| 12 | (14)Vibration Motor |  | (14)Vibration Motor | 1 |  | PCS |  |
| 13 | （15）Battery |  | LITHIUM ION BATTERIES | 1 |  | PCS |  |
|  |  |  | Battery BTB Silicone Case | 1 |  | PCS |  |
|  |  |  | Battery anti-static bag | 1 |  | PCS |  |
| 14 | （16）RF connecting cable(white） |  | 50Ω 123.5mm | 1 |  | PCS |  |
| 15 | （17）RF connecting cable(blue） |  | 50Ω 188mm | 1 |  | PCS |  |
| 16 | (18)Antenna board |  | PCB semi-finished（A1-PCB） | 1 |  | PCS |  |
|  |  |  | waterproof paste | 1 |  | PCS |  |
|  |  |  | Motor sponge | 1 |  | PCS |  |
| 17 | (19)PCB patch semi-finished product |  | A2AE008-0-patch 19191 main（Below speaker BOX） | 1 |  | PCS |  |
| 18 | (20)Speaker BOX |  | 1.14W 7Ω 12*16*2.35 Monomer A | 1 |  | PCS |  |
| 19 | (21)Small board bracket |  | AE009 black with foam | 1 |  | PCS |  |

Note：This table is used only for the name of the annotation but not for the preparation of materials. Please refer to the Price List of OPPO Mobile Phone Original Accessories (for Mainland China)/Price List of Commonly-Used Materials and Accessories (Regions except for Mainland China) for the preparation.

4. Disassembly Steps

4.1 Disassembly Tools

4.2 Preparation

1. The phone must be powered off before disassembly.

2. Way to Power Off:

System Power Off: Press the power key then slide to power off.

4.3 Disassembly Steps

| Step | Content | Figure | Cautions |
| --- | --- | --- | --- |
| 1 | Remove the back cover |  | (1)Avoid heating the camera. Heating requirements:70℃-85℃,5-10min. (2) Use the suction cup to remove the back cover from the bottom of the phone; adjust the suction cup and strength according to the tightness when separating it. |
| 2 | Remove the battery |  | (1)Remove the screws from mainboard bracket (1#, 10pcs)； (2) Inject about 0.2ml alcohol into the gap of the easy-to-tear paper and battery. Slant the phone for 10 seconds. (3) Pull up the other side easy-tore film and repeat the above actions. (4) Place the phone flat, and pull the handle at an angle (greater than 90 °), so that the battery is slightly separated from the battery pull tape. (5) Slowly pull the handle with both hands at the same time, completely separating the battery from the battery pull tape. Note: Avoid bending and deformation of the middle part of the battery during disassembling the battery, otherwise the battery may be damaged. |
| 3 | Remove the camera |  | (1)Rear camera can be disassembled separately for repair after remove the copper foil. (2)Remove the Front camera copper foil. The front camera copper foil and grounded foam must be replaced after repair. (3) Remove the Front camera. |
| 4 | Remove the Small board bracket |  | (1)Add 1 drop of alcohol to the hole of the Small board bracket, and disassemble the Small board bracket from top to bottom. (2) Clean the double-sided tape of the vibration motor and the small board bracket. |
| 5 | Remove the mainboard and the L-FPC |  | Use a disassemble rod and a dust-free cloth to clean the thermal gel after replaced the mainboard or the screen, and apply it during assembly. |
|  | Remove the receiver and the antenna board |  | Use plastic tweezers to loosen the FPC of the receiver, and then disassemble the receiver from the lower right corner. (The double-sided tape of the receiver will be damaged after being disassembled. Pay attention to clean up the residual glue on the middle frame of mobile phone. ) |
|  | Remove the speaker BOX, the A2-antenna board and RF connecting cable |  | First disassemble the FPC on the right side of the speaker to avoid tearing when disassembling the speaker. |
|  | Remove the USB-FPC and the Vibration Motor |  | Pay attention to clean up the double-sided tape of the front camera hole. |

Figure 4.3 All Materials of the Phone

5. Assembly Steps

5.1 Bill of Must-Be-Replaced Materials

| No. | Code | Material | SPEC. | QTY | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | 4881320 | Battery cover double-sided adhesive | AE009 | 1 | Must be replaced after disassembling the back cover |
| 2 | 2930994 | The volume keys | AE009 | 1 | Must be replaced after disassembling the volume key |
| 3 | 2930993 | The power keys bracket | AE009 | 1 | Must be replaced after disassembling the power key |
| 4 | 4881317 | Motor sponge （Small board bracket double-sided tape） | AE009 single-sided adhesive | 1 | Must be replaced after disassembling the small board bracket and damage |
| 5 | 4881086 | battery pull tape | BLP769 transparent | 1 | Must be replaced after removing the battery |
| 6 | 4882565 | Antenna plate double-sided adhesive | AE009 | 1 | Must be replaced after disassembling the Speaker BOX and damage |
| 7 | 4881285 | Front camera fixed double-sided adhesive | AE009 | 1 | Must be replaced after disassembling the front camera |
| 8 | 4881284 | REC conductive copper foil （the copper foil of the receiver in disassembly video.） | AE009 | 1 | Must be replaced after disassembling the mainboard or the receiver |
| 9 | 4881280 | camera conductive copper foil(Front) | AE009 single-sided adhesive | 1 | Must be replaced after disassembling the mainboard or the front camera |
| 10 | 4881657 | Front camera grounded foam | AE009 single-sided adhesive | 1 | Must be replaced after disassembling the mainboard or the front camera and damage |

5.2 Assembly Steps

| Step | Content | Figure | Cautions |
| --- | --- | --- | --- |
| 1 | Install the USB-FPC, the Vibration Motor and the receiver |  | (Pay attention to clean up the double-sided tape beside the receiver and the Vibration Motor.) |
| 2 | Install the A2- antenna board, the speaker BOX and the blue RF connecting cable |  | (First install the blue RF connecting cable on the A2-antenna board, and then install the speaker BOX and A2-antenna board, pay attention to the FPC of the speaker.) |
| 3 | Install the white RF connecting cable, the antenna board and load the card tray |  | After the antenna board is assembled, fix it with the card tray. |
| 4 | Buckle up the BTB of L-FPC and Install the bracket of antenna board |  | Before installing the bracket of antenna board, check the double-sided adhesive between the motor and the bracket of antenna board. |
| 5 | Apply thermal gel and assembly the mainboard |  | Pay attention to clean up the residual glue on the middle frame of mobile phone. Apply thermal gel during assembly. |
| 6 | Assembly the front camera |  | - Do not make a mistake in the position of the pre-fixed screw on the motherboard. - First paste the camera conductive copper foil (Front) on the back of the front camera, and then paste the Front camera grounded foam. - The camera conductive copper foil(Front) and the Front camera grounded foam must be replaced after disassembling the front camera |
| 7 | Assembly the rear camera |  | The camera combination copper foil sheet (Rear) must be replaced after disassembling the mainboard or the front camera and damage. |
| 8 | Buckle the RF cable |  | / |
| 9 | Load the battery |  | The battery pull tape must be replaced after disassembling the battery. |
| 10 | Load the mainboard bracket and back cover |  | After the battery cover double-sided adhesive is attached, press the back cover around with your hand to activate the double-sided adhesive. |

6. Calibration

The following calibration must be performed after disassembling or replacing the phone, Reno4 Pro 5G calibration include Leak Calibration, Finger print optical calibration, TP calibration, Aftersale Triple camera calibration, OIS-Still and Smart power amplifier calibration. After the calibration is completed, the phone needs to be restarted.

After-sales calibration tools:

| The bubble bag | Fingerprint Calibration tool | Chart card |
| --- | --- | --- |

6.1 AfterSale Devices Calibrate

6.1.1 Leak Calibration

Must be calibrated after disassembling the mainboard or the screen, Leak calibration is suitable for mobile phones that has under-screen proximity & light sensor, Affect the brightness adjustment function of the mobile phone screen. enter *#899# in dial interface to enter engineer mode, click AfterSale Devices Calibration , enter the password “6776” to enter the Leak Calibration.

| Must ensure that the upper part of the device is in a completely dark environment, don't cover the phone and then press calibration button. During the calibration process, the screen will automatically switch (red, green, blue, and white) different color interfaces. After pass automatically jump to next interface. |  |  |
| --- | --- | --- |
| enter the password “6776” to enter the Leak Calibration | Use the bubble bag to ensure a dark environment before enter the Leak Calibration | After completed automatically jump to next interface |

6.1.2 Finger print optical calibration

Must be calibrated after disassembling the mainboard, the screen or capacitive touch screen protective film.

| Operation | 1. Click "START" to start calibration. | 2. According to the prompt "please place the yellow rubber ", put the yellow rubber on the fingerprint spot area (the groove is facing down), and click "NEXT" until it prompts "please place the black rubber". | 3. According to the prompt "please place the black rubber head", put the black rubber on the fingerprint spot area (the groove is facing down), click "NEXT" until the "please place the yellow stripe rubber " prompts to remove the black rubber ; |
| --- | --- | --- | --- |
| Icon |  |  |  |
| Operation | 4. According to the prompt “Please place the yellow- striped rubber”, put the yellow stripe rubber (striped down) in the spot area, click “NEXT”; | 5. If it prompts fail, you need to recalibrate the test, or check whether there is a problem with the fingerprint installation. | Notice： 1) Installation and precautions for fixtures can also be found in the knowledge base file: “OPPO Customer Service Center Repair Tool Instruction Manual _V1.0”. 2)The yellow-stripe rubber head placement requirements as shown below. |
| Icon |  |  |  |

After the calibration is completed, restart the phone to make the calibration take effect, and guide the user to delete the original fingerprint and re-enter.

6.1.3 TP calibration

After the Finger print optical calibration is completed，it’s will jump into TP calibration and perform calibration at the same time (no need to click). During the calibration process, a white screen will be displayed on the screen, which is normal. Do not touch the screen during calibration.

Must be calibrated after disassembling the mainboard or the screen, the feedback of the touch screen is not working can also try to calibrate whether it can be solved.

6.1.4 Aftersale Triple camera calibration

Because the rear camera can be disassembled separately, the factory only performs calibration for a single camera. After disassembling the machine in the customer service center, the whole machine needs to be calibrated.

6.1.4.1 Calibration Scenario

1. When the portrait mode is poorly blurred, the after-sales service center needs to calibrate the phone.

2. All devices must be calibrated after the phone has disassembled, or the portrait mode will be poorly blurred.

6.1.4.2 Calibration Tool

After-sales calibration Chart,

As shown in Figure 6.1.4.3:

Figure 6.1.4.3

6.1.4.3 Calibration Method

| After the TP calibration is completed, it will automatically jump to “Aftersale Triple camera calibration” and enters the calibration interface. Please align the yellow box with the edge of the chart (about 30cm~50cm), and then take a Photo. Then, do not operate the phone when the phone enters the computing interface until it displays “Pass” with the calibration completed. If it displays "fail", adjust as prompted and recalibrate until the calibration display passes. main and wide/ main and tele need to be calibrated separately，two calibrations are “pass” is mean the “Aftersale Triple camera calibration” successfully. |  |  |
| --- | --- | --- |
| Calibration interface | Calibration Calculation Process Interface | Calibration PASS result interface |

6.1.5 OIS-Still

After the Aftersale Triple camera calibration is completed，it’s will jump into OIS-Still. When calibrating, put the mobile phone on the repair table and do not operate the mobile phone. After the auto-calibration is successful, it will prompt "test successed" (as Figure 6.1.5.1).

Must be calibrated after disassembling the mainboard or the camera.

6.1.6 Smart power amplifier calibration

After OIS-Still is completed, return manually to enter the Smart power amplifier calibration. After clicking "calibrate"(as Figure 6.1.6.1), the result will be output automatically, and the calibration interface will be automatically exited after completion.

Figure 6.1.5.1                   Figure 6.1.6.1

After the calibration process is completed, the phone must be restarted for the calibration to take effect! After restarting, enter Function Test.

7.  Function Test

7.1 Test Path

Dialing Interface>Enter*#899#>Enter automatic test； The "Confirm" dialog box will pop up after the test complete, if there is no problem choose “Pass”, otherwise choose “Fail”.

7.2 Test Requirements

| No. | Test Items | Test Requirements |
| --- | --- | --- |
| 1 | LCD Test | Observe the red, green, blue, white, black, gray, grayscale and color interface successively; If there are black spots, bright spots and other anomalies, the function is good; |
| 2 | Vibration Test | Get ear close to check the vibration from the motor for 3 seconds. If the motor vibrates normally(No noises or weak vibration), the function is good; |
| 3 | Touch Screen Auto Test | If the interface shows “PASS” and jumps to the next test, the function is good; |
| 4 | Front Camera Preview | Align the rear camera with a white surface and a black surface. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, and there is no bright spot on the screen after aligning the black surface, the camera is good; |
| 5 | Rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is good; |
| 6 | Second rear camera preview | Align the Second rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is good; |
| 7 | third rear camera preview | Align the third rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is good; |
| 8 | Echo test | Testing Main MIC: Blow the air to the MIC hole on the bottom of the phone. If the receiver (earphone) sounds, the function of Main MIC is good; Testing Sub MIC: Blow the air to the sub MIC hole on the top of the phone. If the speaker sounds, the function of Main MIC is good; |
| 9 | Flashlight Test | Open the flashlight. If it can be opened normally with color deviations, the function is good; |
| 10 | Super VOOC Charging | Use the original OPPO adapter and USB cable to charge. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 11 | Headset in-out Plug test | Plug in and out the earphone with the OPPO earphone. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 12 | Sensor Self-test and Calibration | When the phone enters into the Sensor self-calibration interface, the phone must be placed on the desk flatly. Then click the first calibration item. After the test is passed, it will automatically jump to the next test item; |
| 13 | Gyroscope | Take the mobile phone to draw the "8" word graphic. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 14 | MSensor Test | Shake the phone from the left to the right side. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 15 | Proximity-sensor Test | Use the palm to cover the light sensor hole, and the screen will turn green from black. If the interface shows “PASS” and jumps to the next test, the function is good; |
| 16 | Keypad Test | Press the power key and volume key one by one. If keys can be pressed normally and the interface jumps to the next test, the function is good; |
| 17 | Finger print auto Test | If the interface shows “PASS” and jumps to the next test, the function is good; |
| 18 | Media test | Click the “20-4K（-3db)signal” to test the sound of the media function for 5s. If there is no silence, weak sound, noises, and breaking sound, the interface shows “PASS” and jumps to the next test, the function is good; |
| 19 | Call test | If the phone has been installed with the operator's SIM card, dial the corresponding operator's phone number. If no card is installed, call the 112 to test (Calling the Operator is recommended). If the call can be performed normally and the receiver is without noises or weak sounds, the function is good; |
| 20 | NFC Test | Close the phone of the rear camera area to the NFC card reader. If the signal light of the card reader changes from red to green with “Beep” sound, the interface shows “PASS” and jumps to the next test, the function is good. |
| 21 | Laser focus | Aim the mobile phone at any object, move the mobile phone, and change the distance between the mobile phone and the object. test the pass when After the mobile phone detects the distance change. |

After the above 21 items are tested, they will prompt to the test result interface. If the test PASS, project will show “√”; If the test FAIL, project will show “×” as shown below.

Figure 7-1                      Figure 7-2

8. General Cautions

- Reno4 Pro 5G standard version have only one screw material, do not make a mistake in the position of the pre-fixed screw on the motherboard;

- When disassembling (front/rear) camera and fingerprint module, pay attention to protect the camera lens with low mucosa;

- First disassemble the FPC on the right side of the speaker to avoid tearing when disassembling the speaker;

- If it involves replacing the screen assembly, the main board or cleaning the original thermal gel, it is necessary to apply the thermal conductive gel according to the above installation steps;

- For Reno4 Pro 5G Disassemble and assemble Guidance video Please visit the OPPO knowledge base.

| Version | Date | Draft | Review | Approve | Modified content |
| --- | --- | --- | --- | --- | --- |
| V1.0 | 2020-07-30 | Chenjinling | Benny | Wong | 1.First edition |