OPPO A73 5G Service Manual _V1.0

Confidential

Content

1.	Guidance	3

1.1 Warnings	3

1.2 Cautions	3

2.	General Repair Information	4

3.	Phone Structure Diagram	5

3.1 Product Appearance	5

3.2 Phone Structure Explosive View	6

3.3 Explosive Map BOM (The number in brackets is the Figure 3-2-1 number)	6

4.	Disassembly Steps	10

4.1 Disassembly Tools	10

4.2 Preparation	10

4.3 Disassembly Steps	11

5.	Assembly Steps	17

5.1 Bill of Must-Be-Replaced Materials	17

5.2 Assembly Steps	17

6.	Calibration	23

6.1Calibration Scenario	23

6.2 Calibration Tool	23

6.3 Calibration Method	24

7.	Function Test	25

7.1 Test Path	25

7.2 Test Requirements	25

8.	General Cautions	28

- Guidance

The purpose of this document is only to guide OPPO technicians to carry out maintenance service to OPPO products. The content shall be keep in confidential, only avail be able to OPPO authorized service centers, and cannot be provided to third parties without authorization. Please follow the regulations and the guidance for maintenance service. For any problems, please contact China HQ in time.

1.1 Warnings

1. Care must be taken on installation in vehicles fitted with electronic engine management systems and ABS (Anti–skid Braking Systems). Under certain fault conditions, emitted RF energy can affect their operation. If necessary, consult the vehicle dealer/manufacturer to determine the immunity of vehicle electronic systems to RF energy.

2. Phones must not be operated in areas likely to contain potentially explosive atmospheres, e.g. petrol stations (service stations), blasting areas etc.

3. Operation of any radio transmitting equipment, including cellular telephones, may interfere with the functionality of inadequately protected medical devices. Consult a physician or the manufacturer of the medical device if you have any questions. Other electronic equipment may also be subject to interference

1.2 Cautions

1. Repair services and calibrations must be undertaken by qualified personnel only.

2. Use only approved components as specified in the parts list.

3. Ensure all components, modules screws and insulators are correctly re-fitted after servicing and alignment.

4. Electrostatic discharge is the main cause of damage to sensitive components of electronic products. Service centers must operate in accordance with OPPO's requirements for ESD protection. Ensure all work is carried out at an ESD workstation and that an ESD wristband is worn.

- General Repair Information

1. Ensure all work is carried out at an ESD workstation and that an ESD wristband is worn before starting maintenance.

2. Wear ESD gloves to avoid oil stains and fingerprints.

3. Use protective films to protect the display, camera, camera lens to prevent dust and scratches.

4. Use a clean cloth, ESD brush and alcohol (Concentration above 95%) for appearance cleaning. Do not use other items (such as an eraser) for cleaning to prevent the protective layer from being scratched, which may lead to oxidation and corrosion.

5. Faulty welded mechanical parts (except shield cover and shield frame parts) can only be replaced and not repairable.

6. Use accessories provided by OPPO.

7. Check the contacts or solder joints of devices that may cause simple faults (e.g. soldered interfaces or switches). Re-weld if necessary (only for service centers where lead-free soldering is available) and clean the flux remaining after soldering.

8. Use the equipment provided by OPPO to test the phone (e.g. When the charging function of the device is abnormal, do use the original OPPO adapter to test to ensure the accuracy of the test results).

9. Fill in the fault phenomenon code and fault reason code according to the actual situation, and accurately enter the replacement material code in the CRM system.

10. For more information about products,

Mainland China please visit:

http://172.16.41.122:8888/kbaseui-dev/main.do

Regions except for Mainland China please visit:

ftp://ftpex.oppo.com:919 or https://oppo-knowledge.custhelp.com

- Phone Structure Diagram

3.1 Product Appearance

As shown in Figure 3-1-1

Figure 3-1-1

(Note: ⑩Screen facing down: rear main camera: 16MP (upper left) + rear wide angle camera: 8MP (lower left) + rear portrait camera: 2MP (upper right).)

3.2 Phone Structure Explosive View

As shown in Figure 3-2-1

Figure 3-2-1

3.3 Explosive Map BOM (The number in brackets is the Figure 3-2-1 number)

| No. | Material Name(After-sales) | Physical Figure | Sub-item name | QTY | Unit |
| --- | --- | --- | --- | --- | --- |
| 1 | Battery cover |  | (1)Battery cover | 1 | PCS |
| 2 | Machine-tapping screw |  | (2)Machine-tapping screw（There are two kinds of screws in the machine：2+17pcs） | 19 | PCS |
| 3 | Middle frame |  | (3)Camera lens | 1 | PCS |
|  |  |  | (4)Camera decorative ring | 1 | PCS |
|  |  |  | (5)Middle frame | 1 | PCS |
| 4 | Rear Main Camera |  | (6)Rear main camera | 1 | PCS |
|  |  |  | Main camera conductive fabric | 1 | PCS |
| 5 | Rear Portrait Camera |  | (7)Rear Portrait Camera | 1 | PCS |
|  |  |  | Rear camera conductive copper foil | 1 | PCS |
|  |  |  | Flash thermal conductive silicon | 1 | PCS |
| 6 | Mainboard |  | (8)PCB semi-finished | 1 | PCS |
|  |  |  | Light perception rubber | 1 |  |
| 7 | Volume key FPC |  | (9)FPC patch semi-finished product | 1 | PCS |
| 8 | C-FPC |  | (10)FPC patch semi-finished product | 1 | PCS |
| 9 | Screen |  | (13)Capacitive touch screen protectivase film | 1 | PCS |
|  |  |  | (11)Display assembly | 1 | PCS |
|  |  |  | FPC patch semi-finished product（Power key FPC） | 1 | PCS |
|  |  |  | FPC patch semi-finished product（Volume key FPC） | 1 | PCS |
|  |  |  | Vibration motor | 1 | PCS |
|  |  |  | waterproof paste | 1 | PCS |
| 10 | SIM card tray |  | (12)SIM card tray | 1 | PCS |
| 11 | RF cable |  | (14)RF cable(white+black) | 2 | PCS |
| 12 | Battery |  | (16)LITHIUM ION BATTERIES | 1 | PCS |
|  |  |  | Battery anti-static bag | 1 | PCS |
| 13 | Antenna board |  | (15)PCB semi-finished （The material without USB silicone sleeve ） | 1 | PCS |
| 14 | Speaker |  | (17)Speaker | 1 | PCS |
|  |  |  | Speaker graphite sheet | 1 | PCS |
|  |  |  | Speaker antenna spring | 1 | PCS |
| 17 | Fingerprint Sensor Module |  | (18)Fingerprint Sensor Module | 1 | PCS |
| 18 | Rear Wide Angle Camera |  | (19)Rear Wide Angle Camera | 1 | PCS |
|  |  |  | Rear camera conductive copper foil | 1 | PCS |
|  |  |  | Flash thermal conductive silicon | 1 | PCS |

Note：This table is used only for the name of the annotation but not for the preparation of materials. Please refer to the Price List of OPPO Mobile Phone Original Accessories (for Mainland China)/Price List of Commonly-Used Materials and Accessories (Regions except for Mainland China) for the preparation.

- Disassembly Steps

4.1 Disassembly Tools

4.2 Preparation

1. The phone must be shut down with the power option before disassembling.

2. Way to Shut Down:

System Power Off: Press the power key then slide to power off.

4.3 Disassembly Steps

| Step | Content | Figure | Cautions |
| --- | --- | --- | --- |
| 1 | Remove the battery cover |  | (1) Avoid heating the camera. Heating requirements: 70 ° C -80 ° C for 5-10 mins; (2) Use the suction cup to remove the battery cover from the bottom of the phone; adjust the suction cup and strength according to the tightness when separating it. |
| 2 | Remove the battery |  | (1) Remove the screws from middle frame (1#, 17pcs)； (2) Remove the middle frame from the lower right corner. (pay attention to open the fingerprint BTB button first.) (3)After tearing off the battery pull tape , add 0.2ml of alcohol to the upper and lower sides of the handle, and stand the phone for 10 seconds until the alcohol spreads; (4) Place the phone flat and pull it diagonally. Make the angle between the handle position and the battery are larger than 90°, and slowly pull the battery off. Note: Since the adhesiveness of the OPPO A73 5G battery pull tape is strong, avoid bending and deformation of the middle part of the battery during assembly. |
| 3 | Remove the rear & front cameras |  | (1) The flash thermal conductive silicon(white) and the rear camera conductive copper foil must be replaced after repair the rear wide angle camera and the rear portrait camera. (2) The front camera conductive copper foil must be replaced after repair the front camera. |
| 4 | Remove the mainboard and clean the thermal gel |  | (1) Note the mainboard screw and speaker screw need to be removed firstly(2#, 2pcs); (2) Remove the main board from the lower right corner. (3) The thermally gel must be cleaned and replaced after replacing the mianboard or screen . |
| 5 | Tear off the foam on the bottom side and right side of the battery compartment、 remove the C-FPC |  | / |
| 6 | Remove the volume key FPC and the power key FPC |  | / |
| 7 | Remove the RF cables, speaker and antenna board |  | (1)The RF connector should be pulled vertically to prevent damage to the socket; (2)Take out the speaker from the right side; (3)Grasp the left side of the antenna board with plastic tweezers and take it out. |
| 8 | Remove the vibration motor and the receiver |  | / |
| 9 | Remove the PCB semi-finished(Flash board) and the fingerprint sensor module |  | / |

Figure 4-3 All Materials of the Phone

- Assembly Steps

5.1 Bill of Must-Be-Replaced Materials

| No. | Code | Material | SPEC. | QTY | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | 4874354 | Battery pull tape | BLP673 transparent | 1 | Must be replaced after removing the battery. |
| 2 | 4882315 | The main board is covered with waterproof foam rubber | FA219 | 1 | Must be replaced after removing the battery cover. |
| 3 | 4882328 | Camera decorative ring sealing sponge | FA219 PORON single-sided adhesive | 1 | Must be replaced after removing the battery cover. |
| 4 | 4882337 | Fingerprint PET sheet | FA219 single-sided adhesive | 1 | Be used during assembly middle frame. |
| 5 | 4882331 | Front camera conductive copper foil | FA219 single-sided adhesive | 1 | Must be replaced after disassembling the front camera or the mainboard. |
| 6 | 4882332 | Rear camera conductive copper foil | FA219 single-sided adhesive | 1 | Must be replaced after disassembling the rear camera or the mainboard. |
| 8 | 4882336 | Flash thermal conductive silicon | FA219 | 1 | Must be replaced after disassembling the rear camera or the mainboard. |

5.2 Assembly Steps

| Step | Content | Figure | Cautions |
| --- | --- | --- | --- |
| 1 | Install the vibration motor，receiver， volume key FPC and power key FPC |  | (1) Keep the direction of the shrapnel facing up when installing the receiver. Press it gently with tweezers without a bulge； （2）Do not install the volume key when installing the volume key FPC. |
| 2 | Inject the thermal gel and install the mainboard |  | (1) Don't forget to add thermal gel. (2)Screw up the mainboard screw（2#，1pcs) |
| 3 | Instal the front camera and rear cammeras |  | (1) The front camera conductive copper foil must be replaced after repair the front camera. (2) The flash thermal conductive silicon(white) and the rear camera conductive copper foil must be replaced after repair the rear wide angle camera and the rear portrait camera. |
| 4 | Instal the RF cable to the mainboard |  | (1)Instal the volume key and paste it tightly. |
| 5 | Instal the antenna board, speaker and instal the RF cables to the antenna board |  | (1)Screw up the speaker screw（2#，1pcs). |
| 6 | Instal the the main FPC and the C-FPC |  | (1)Buckle the BTB connectors and paste the FPC well. |
| 7 | Instal the battery |  | (1) Battery pull tape and battery extracting sticker must be replaced new one. |
| 8 | Stick the front camera conductive copper foil, the front camera conductive copper foil and the flash thermal conductive silicon |  | / |
| 9 | Instal the PCB semi-finished(Flash board) and the fingerprint sensor module |  | (1) When installing the fingerprint sensor module, first paste the fingerprint PET sheet as an auxiliary, then buckle the fingerprint BTB, and then instal the middle frame. After the repair is completed, pay attention to tear off the fingerprint PET sheet. |
| 10 | Instal the middle frame, battery cover and SIM card tray |  | (1)Stick the battery cover double-sides tape and the camera decorative ring sealing sponge, and than instaling the battery cover and press it tightly. |

- Calibration

The following fingerprint calibration must be performed after disassembling or replacing the phone,  the calibration item of OPPO A73 5G is after-sales calibration

6.1Calibration Scenario

1. When the portrait mode is poorly blurred, the after-sales service center needs to calibrate the phone.

2. All devices must be calibrated after the phone has disassembled, or the portrait mode will be poorly blurred.

Figure 6-2-1

6.2 Calibration Tool

After-sales calibration Chart, As shown in Figure 6-2-1

6.3 Calibration Method

| Please align the yellow box with the edge of the chart, and then take a Photo. Then, do not operate the phone when the phone enters the computing interface until it displays “Pass” with the calibration completed. |  |  |
| --- | --- | --- |
| Calibration interface | Calibration Calculation Process Interface | Calibration PASS result interface |

At this point, the calibration process is over and the phone must be restarted after the calibration process is completed for the calibration to take effect! After rebooting, enter the functional test.

- Function Test

7.1 Test Path

Dialing Interface>Enter*#899#>Enter automatic test； The "Confirm" dialog box will pop up after the test complete, if there is no problem choose “Pass”, otherwise choose “Fail”.

7.2 Test Requirements

| No. | Test Items | Test Requirements |
| --- | --- | --- |
| 1 | LCD test | Observe the red, green, blue, white, black, gray, grayscale and color interface successively; If there are black spots, bright spots and other anomalies, the function is sound; |
| 2 | Vibration test | Get ear close to check the vibration from the motor for 3 seconds. If the motor vibrates normally(No noises or weak vibration), the function is sound; |
| 3 | Touch screen auto Test | If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 4 | Front camera preview | Align the front camera with a white surface and a black surface. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, and there is no bright spot on the screen after aligning the black surface, the camera is sound; |
| 5 | Rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is sound; |
| 6 | Second rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is sound; |
| 7 | Third rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is sound; |
| 8 | Echo test | Testing Main MIC: Blow the air to the MIC hole on the bottom of the phone. If the receiver (earphone) sounds, the function of Main MIC is sound; Testing Sub MIC: Blow the air to the sub MIC hole on the top of the phone. If the speaker sounds, the function of Main MIC is sound; |
| 9 | Flash-light test | Open the flashlight. If it can be opened normally with color deviations, the function is sound; |
| 10 | Generic Charger | Use the original OPPO adapter and USB cable to charge. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 11 | QC Charger | Use the original OPPO QC adapter and USB cable to charge. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 12 | Headset in-out plug test | Plug in and out the earphone with the OPPO earphone. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 13 | Sensor self-test and calibration | When the phone enters into the Sensor self-calibration interface, the phone must be placed on the desk flatly. Then click the first calibration item. After the test is passed, it will automatically jump to the next test item; |
| 14 | Gyroscope | Take the mobile phone to draw the "8" word graphic. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 15 | M-Sensor | Shake the phone from the left to the right side. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 16 | Proximity Test | Use the palm to cover the light sensor hole, and the screen will turn green from black. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 17 | Keypad test | Press the power key and volume key one by one. If keys can be pressed normally and the interface jumps to the next test, the function is sound; |
| 18 | Finger print auto test | If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 19 | Media test | Click the “20-4K（-3db)signal” to test the sound of the media function for 5s. If there is no silence, weak sound, noises, and breaking sound, the interface shows “PASS” and jumps to the next test, the function is sound; |
| 20 | Call test | If the phone has been installed with the operator's SIM card, dial the corresponding operator's phone number. If no card is installed, call the 112 to test (Calling the Operator is recommended). If the call can be performed normally and the receiver is without noises or weak sounds, the function is sound; |
| 21 | NFC test | Close the phone of the rear camera area to the NFC card reader. If the signal light of the card reader changes from red to green with “Beep” sound, the interface shows “PASS” and jumps to the next test, the function is sound. |

After the above 21 items are tested, they will prompt to the test result interface. If the test PASS, project will show “√”; If the test FAIL, project will show “×” as shown below.

Figure 7-2-1                       Figure 7-2-2

- General Cautions

1. OPPO A73 5G standard version has two different screw material (large, small) , No.1 large screws has 17pcs total, No.2 small screws has 2pcs total . Do Not Mixing screws;

2. When disassembling (front/rear) camera and fingerprint module, pay attention to protect the camera lens with low mucosa;

3. When installing the fingerprint sensor module, first paste the fingerprint PET sheet as an auxiliary, then buckle the fingerprint BTB, and then instal the middle frame. After the repair is completed, pay attention to tear off the fingerprint PET sheet;

4. If it involves replacing the screen assembly, the main board or cleaning the original thermal gel, it is necessary to apply the thermal conductive gel according to the above installation steps;

5. For OPPO A73 5G Disassembly Video Please visit:

（1）FTP: Overseas Costumer Service/OPPO Technical Service/Product Material/OPPO A73 5G(CPH2161)

----(OPPO A72 5G&OPPO A72n 5G&OPPO A73 5G Disassembly and Assembly Guidance Video)（2）Video link: https://fileload.coloros.com/29717OPPO%20A72%205G%26OPPO%20A72n%205G%26OPPO%20A73%205G%20Disassembly%20and%20Assembly%20Guidance%20Video.mp4

| Version | Date | Draft | Review | Approve | Modified content |
| --- | --- | --- | --- | --- | --- |
| V1.0 | 2020-10-23 | Xu YiTing | Li BinBin | Huang GuoXin | 1st Version |