Version record

| Version | Issuance Date | Revision | Drafted | Reviewed | Approved |
| --- | --- | --- | --- | --- | --- |
| V1.0 | 2018-06-01 | First Version | Hu Yun | Chen Wei | Wang Lei |
| V1.1 | 2020-09-25 | Added IoT product content | Hu Yun | Li BinBin | Huang GuoXin |
| V1.2 | 2020-10-26 | Modify the judgment of screen crack/afterimage | Hu Yun | Li BinBin | Huang GuoXin |

Content

1.	Purpose	2

2.	Scope	2

3.	Principles for Replacement	3

4.	Principles for Warranty Judgment	3

5.	Warranty judgment for handset	4

5.1 Appearance Inspection	4

5.2 Electronic Components Inspection	13

5.3 Liquid Inspection	14

5.4 Software Inspection	15

5.4.1 Check ROM	15

5.5 Check ROOT status	16

6.1 Warranty Judgment for power bank	17

6.1.1 Appearance Inspection—IW (in-warranty)	17

6.1.2 Basic function Inspection (power bank)	19

6.1.3 Liquid Inspection (power bank)	19

6.1.4 Software Detection (power bank)	19

6.2 Warranty Judgment for neck-mounted earphones	20

6.2.1 Appearance Inspection	20

6.2.2 Basic function inspection (Neck-mounted earphones)	24

6.2.3 Liquid Detection (Neck-mounted earphones)	24

6.2.4 Software Detection (Neck-mounted earphones)	24

6.3 Warranty Judgment for Selfie Stick	25

6.3.1 Appearance Inspection	25

6.3.2 Basic function inspection (Selfie stick)	27

6.4 Warranty Judgment for Game Pad	28

6.4.1 Appearance Inspection	28

6.4.2 Basic function inspection (game pad)	29

6.5 Warranty Judgment for OPPO Watch	30

6.6 Warranty Judgment for TWS Headphones	31

6.7 Warranty Judgment for Accessories	32

7. Warranty Judgment for Accessories	32

- Purpose

The guidance is issued to guide the OPPO service centers and authorized service cooperation agencies to judge the repair of OPPO products more accurately, unify the understanding of the warranty policy, also provide a reference for the HQ staff to check the bad materials return from each service center.

- Scope

All OPPO mobile phones and IoT products

- Principles for Replacement

  - Definition

DOA - dead on arrive: Phones are found to fail in the first opening of the cartons or color-boxes before being sold

DAP – dead after purchase: Phones with non-human damage are guaranteed for return or replacement in line with the conditions of return or replacement.

  - DOA&DAP Eligibility Condition and Flow

| Fault phenomenon Type | DOA | DAP |
| --- | --- | --- |
| With good appearance and normal functions | × | × |
| With good appearance but abnormal functions | √ | √ |
| With slightly defective appearance but normal functions | √ | × |
| With slightly defective appearance and abnormal functions | √ | √ |
| “√”means that the replacement is available; “×”means that the replacement is unavailable; |  |  |

- Principles for Warranty Judgment

| Item | OW(out of warranty) Condition |
| --- | --- |
| Out-of-Warranty for the phone | (1)The phone is not in warranty period； (2) The phone operated beyond the permission or the requirement of the User Manual; (3) The phone disassembled by users itself, any non-OPPO official or non-OPPO authorized service providers; (4) The phone damaged destructively by accidents, misuse, fire, earthquake or other external factors. (5) The phone was suffered by water damage and the user does not agree to pay for the replacement of all the damaged components. (the detail will be stated in the Liquid Inspection); (6) The IMEI has been tampered. |
| Note: After the phone is replaced, the return phone (malfunction) must be attached with complete accessories. If the machine head can only be returned, special explanation is required. IW=In Warranty, OOW=Out of Warranty. |  |

- Warranty judgment for handset

## 5.1 Appearance Inspection

Each of the following inspection item must be carefully checked, and all items must meet the warranty conditions before the phone can be warranted.

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Dust into the camera |  | IW |
| Camera lens missing |  | OOW (IW If the lens is only degummed) |
| Damage of SIM card holder |  | IW |
| Battery Swelling |  | Warranty for the whole device (Quality Record) |
| Foreign color dots on the battery cover |  | IW |
| There is glue overflow at the connection between the screen and the middle frame |  | IW |
| Decoration/ LOGO Fall off the back cover |  | IW |
| Volume button, power button or other physical buttons missing |  | Warranty for the button |
| Flaking paint-off - a flaw in a production process |  | Warranty for paint-off components. |
| Scuffed to flake paint | Scuffed to flake paint | OOW |
| Destruction damage and deformation of the whole phone; | The mobile phone has been crushed by wheels or damaged destructively, such as serious deformation, fracture or missing of the middle frame. ※Explanation: If the phone is damaged destructively, the whole machine will not warranted. | OOW (The phone is unable to be repaired.) |
| The bending of the whole phone; | Frame bending: The machine with bending deformation more than 0.5mm is not recommended to be warranted. Deformation of the whole machine may cause damage to the main board. Explanation: the thickness of 0.5mm is equivalent to the thickness of 6 sheets of A4. The outsourcer needs to install a plug gauge. | According to the measurement results |
| Appearance Damage |  | The damaged components are out of warranty. |
| USB port/earphone port or SIM-card slot pot is damaged or deformed |  | The damaged components are out of warranty. |
| SIM card tray lost or human damaged |  | OOW |
| The outside appearance is dyed |  | The appearance components are out of warranty. |
| Cover Deformation |  | The deformed components are out of warranty. |
| UV screen protect film residual glue |  | OOW |

## 5.2 Screen Inspection

| Type | Description | Warranty Status |
| --- | --- | --- |
| The screen (touch screen or display screen) is cracked with obvious stress concentration points. |  | OOW |
| Screen malfunction, slight fracturing or missing of the screen edge (plastic frame), but no damage to the screen body |  | IW |
| Screen Cracking |  | OOW |
| Low glue-intensity of the screen (The screen is open without broken or missing.) |  | IW |
| Liquid Leakage in the screen |  | IW |
| Screen oxidation layering (The local or entire screen discolored.) |  | IW |
| Water damage or artificial water damage to the screen |  | OOW |
| Screen afterimage |  | IW |
| Dust inside the screen |  | IW |
| Black spots on the edge of the screen, bumps and scratches, but no cracks on the screen |  | IW |
| The screen shows lines, with slight scratches or knocks, no cracks |  | IW |

## 5.3 Electronic Components Inspection

| Type | Description | Warranty Status |
| --- | --- | --- |
| The shield cover is damaged |  | OOW |
| The components are dampened or corroded, and there is obvious accumulation of salt crystals. |  | OOW |
| The mother board is bended (deformed) |  | OOW |
| The component are falling out from the mother board |  | OOW |
| Anti-disassembly label (fragile sticker) is broken If the phone is disassembled, the whole machine will be no longer warranted. |  | OOW |

## 5.4 Liquid Inspection

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| The phone is damaged by liquid |  | OOW |

## 5.5 Software Inspection

The mobile phone has been flashed the unofficial ROM or ROOT, or suffered other software problems:

a) If the problem can be solved for users through flashing or other software processing methods, the problem can be solved for free.

b) If the system cannot work normally after being flashed to recover, the phone is judged as “hardware damaged from ROOT” and it can be free for the maintenance but cannot DOA or DAP.

c) If it is not possible to judge whether the device is root, the device can be free for the maintenance but cannot DOA or DAP.

### 5.5.1 Check ROM

| Type | Description | Warranty Status |
| --- | --- | --- |
| Being flashed the unofficial ROM | OPPO Official System：Color OS system | Being flashed the unofficial ROM (can be free for the maintenance but cannot DOA or DAP) |

### 5.5.2 Check ROOT status

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Check whether the phone is rooted | Without Root Rooted! 1. Input the *#808# in the dial of standby interface, and all the models with the system before Android 5.0 Version will enter the engineering mode to check the "Root Status detection". The display interface of the root status is as follows: 2. The phone with the system after Android 5.0 Version: Press the power key and volume down button at the same time to enter the Recovery Mode----Simplified Chinese-- Disk Detection. If the phone is without being Rooted, the interface will appear "Disk Detection". | If the phone detected is Rooted (can be free for the maintenance but cannot DOA or DAP)) |

6. Warranty Judgment for IoT Products

## 6.1 Warranty Judgment for power bank

### 6.1.1 Appearance Inspection—IW (in-warranty)

Each of the following inspection items must be carefully checked, and all items must meet the warranty conditions before the device can be warranted.

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Low glue-intensity of the shell |  | IW |
| Destructive damage and deformation of the whole device |  | OOW |
| The USB jack is damaged or deformed. There are foreign bodies that cannot be removed in the USB jack. |  | OOW |
| The battery is removed privately or damaged |  | OOW |
| The device was disassembled privately |  | OOW |
| The bending of the whole device | The device with bending thickness more than 0.5mm is out of warranty. Deformed appearance may cause damage to the battery. | According to the measurement results |

### 6.1.2 Basic function Inspection (power bank)

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Unable to charge the power bank | - Unable to charge ordinary power bank with OPPO ordinary adapter - Unable to charge VOOC power bank with OPPO VOOC adapter - Unable to charge SUPER VOOC power bank with OPPO SUPER VOOC adapter | IW |
| Unable to charge the phone with power bank | (1) Ordinary power bank cannot charge the phone (2) VOOC power bank cannot charge or cannot VOOC charge the phone (3) SUPER VOOC power bank cannot charge or cannot super VOOC charge the phone | IW |
| The charging indicator is abnormal | (1) The charging indicator doesn't work (2) Unable to switch the corresponding charging mode (VOOC/super VOOC) (3) Other functional malfunctions | IW |
| Abnormal charging | - Serious heat during charging - Slow charging - Fast power consumption | IW |

### 6.1.3 Liquid Inspection (power bank)

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Damaged by liquid | There is an abnormal sound when shaking the power bank, and the USB jack has obvious water stained and rusted. | OOW |
|  | Normal appearance, cannot judge whether it is damaged by liquid | IW |

### 6.1.4 Software Detection (power bank)

If still face abnormal charging issue after replaced new USB cable or adapter, try to flash the latest version software for the power bank. If the problem can be solved for users through flashing software, help users deal with it for free.

## 6.2 Warranty Judgment for neck-mounted earphones

### 6.2.1 Appearance Inspection

Each of the following inspection items must be carefully checked, and all items must meet the warranty conditions before the device can be warranted.

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| The silicone plug of the earphone has a dimensional deviation and is easy to fall off. |  | IW |
| Earphone speaker cracking |  | OOW |
| Decorative sheet cracking/deformation |  | IW |
| The earphone wire is stretched to deform or extruded to fracture |  | OOW |
| Earphone main bracket break |  | OOW |
| Rupture in button protection silicone sleeve |  | OOW |
| Multifunction button missing |  | OOW |
| USB jack silicone plug break |  | OOW |
| Earphone silicone plug missing |  | OOW |
| Earphone silicone plug deformation, cracking |  | OOW |
| The jack is damaged or deformed. /There are foreign bodies that cannot be removed in the jack. |  | OOW |
| The outside appearance staining/paint flaking/ scratching |  | OOW |
| Deformation of earphone body |  | OOW |
| The device was disassembled privately |  | OOW |
| Privately disassembled then glued together |  | OOW |
| Earphone degumming |  | IW |

### 6.2.2 Basic function inspection (Neck-mounted earphones)

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Abnormal sound | (1) No sound from the speaker, low sound, noise (2)Different sound levels between two speakers (3)No sound from the microphone, low sound, noise | IW |
| The other function malfunction | 1. Bluetooth is not compatible (not connected, pairing failed, disconnected after connection) 2. Unable to charge, serious heating when charging, fast power consumption 3. Buttons malfunction 4. Abnormal charging indicator 5. Call function malfunction 6. APP compatibility issues | IW |

### 6.2.3 Liquid Detection (Neck-mounted earphones)

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Damaged by liquid | There is an abnormal sound when shaking the power bank, and the USB jack has obvious water and rust. | OOW |
|  | Normal appearance, cannot judge whether it is damaged by liquid | IW |

### 6.2.4 Software Detection (Neck-mounted earphones)

If the above problems can be solved for users through flashing software, help users deal with it for free.

## 6.3 Warranty Judgment for Selfie Stick

### 6.3.1 Appearance Inspection

Each of the following inspection items must be carefully checked, and all items must meet the warranty conditions before the device can be warranted.

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| The remote controller is not firmly installed and easy to fall off |  | IW |
| The selfie stick automatically pulls back after being pulled out |  | IW |
| Plastic buckle fails, connecting rod falls off |  | OWW |
| Tripod deformed/ rupture |  | OWW |
| Phone clamp deformed/ rupture |  | OWW |
| The outside appearance staining/paint flaking/ scratching |  | OOW |
| Selfie stick deformation/ break |  | OOW |
| Remote controller lost due to users’ reasons |  | OOW |
| Remote controller socket rupture/deformation |  | OOW |

### 6.3.2 Basic function inspection (Selfie stick)

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Remote controller malfunction | (1) The remote controller cannot connect to the phone (2) Can't take pictures remotely | IW |

6.3.3Liquid Detection (Selfie stick)

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Water stained in remote controller |  | OOW |

## 6.4 Warranty Judgment for Game Pad

### 6.4.1 Appearance Inspection

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Good appearance but abnormal functions |  | IW |
| The button/joystick was forcibly removed by external force |  | OOW |
| Disassemble privately, missing screws |  | OOW |
| The game pad is split in half |  | OOW |
| Button missing |  | OOW |

### 6.4.2 Basic function inspection (game pad)

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Cannot power on | No liquid damage, cannot power on | IW |
| Cannot be charged | Cannot be charged with the rated charger. | IW |
| Bluetooth connection | Bluetooth connection is not available when used on any device. | IW |
| Playing A Game | After the Bluetooth is connected, the game cannot be operated through the game pad | IW |

## 6.5 Warranty Judgment for OPPO Watch

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Back shell damaged by external force |  | OOW |
| The screen is damaged by external force |  | OOW |
| The strap is cut |  | OOW |
| Liquid entering the product | / | OOW |
| The charging pin is broken | / | OOW |
| Separation of human factors from screen and middle frame | / | OOW |
| The strap buckle comes off and is missing | / | IW |
| Watch buttons fall off | / | IW |

## 6.6 Warranty Judgment for TWS Headphones

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| The earphone rubber plug is not assembled well | / | IW |
| The headphone body (including the charging case) is cracked and lifted |  | 1. Obvious signs of external force, OOW 2.No obvious signs of external force, IW |
| The charging pin is broken |  | 1. Obvious signs of external force, OOW 2.No obvious signs of external force, IW |
| Headphone speaker cracked | / | OOW |
| The USB interface is damaged or deformed / There is a foreign object in the interface that cannot be removed |  | OOW |
| Shell staining, paint peeling, scratches | / | OOW |
| Shell deformation | / | OOW |
| The shell was disassembled or glued privately | / | OOW |

## 6.7 Warranty Judgment for Accessories

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| Smart band prong damaged |  | OOW |

# 7. Warranty Judgment for Accessories

The accessories including the adapter, data cable, and earphone can be individually replaced after the appearance detection according to the warranty policy.

| Type | Descriptions | Warranty Status |
| --- | --- | --- |
| The plug is deformed or bended |  | OOW |
| The plastic at the plug is cracked |  | OOW |
| The line body is cracked |  | OOW |
| The earphone plug is deformed or broken |  | OOW |
| The plug is burned out |  | IW |
| The connection between the plug and the wire body is short-circuited and burned out |  | IW |
| The earphone wire is stretched to deform or extruded to fracture |  | OOW |
| The insulating adhesive of the earphone jack is damaged or deformed; |  | OOW |
| The dust cloth of the speaker is punctured, and there is obvious oil and other liquid blocking the sound hole. |  | OOW |
| Adapter | - Man-made damage, liquid damage, private disassembly, etc.; 2. Scan the two-dimensional code above the adapter to check the authenticity. If it is a fake one, the adapter will be out of warranty. | OOW |
| Adapter burnt out |  | IW |
| Adapter burnt out |  | IW |

1. If the USB cable of the IoT product is damaged, the user can be informed that the they can use the normal charging, VOOC, SUPER VOOC USB cable corresponding to the original OPPO mobile phone accessories for charging;

2. If there is any other item without being listed, contact the factory technology support group;

3. All the "IW" judgment for the failures listed above are only repaired free of charge during the warranty period, and the warranty time is implemented in accordance with local policies. (Battery swelling problems can be repaired free of charge outside the warranty period).

The guidance is executed from the date of the issuance! The final explanation right is reserved by Global Service Department Technical Support Team.