OPPO Reno5 Pro 5G & Find X3 Neo Service Manual_V1.0

Confidential

Contents

1. Guidance	4

1.1 Warnings	4

1.2 Cautions	4

2. General Repair Information	5

3. Phone Structure Diagram	6

3.1 Product Appearance	6

3.2 Phone Structure Explosive View	7

3.3 Explosive Map BOM	7

4. Disassembly Steps	11

4.1 Disassembly tools required as shown below:	11

4.2 Preparation	11

4.3 Precautions for disassembly	12

5. Assembly Steps	13

5.1 Materials are required to be replaced after disassembly	13

5.2 Precautions for assembly	14

6. Calibration	15

6.1 Leak Calibration	15

6.1 TP Calibration	16

6.1 Fingerprint optical calibration	16

6.2 After-sale Triple Camera Calibration	17

6.2.1 Calibration Scenario	17

6.2.2 Calibration Tool	17

6.3.3 Calibration Method	18

7. Function test	18

7.1 Test Path	18

7.2 Test Requirements	19

8. Cautions	21

1. Guidance

The purpose of this document is only to guide OPPO technicians to carry out maintenance service to OPPO products. The content shall be keep in confidential, only avail be able to OPPO authorized service centers, and cannot be provided to third parties without authorization. Please follow the regulations and the guidance for maintenance service. For any problems, please contact China HQ in time.

1.1 Warnings

1. Care must be taken on installation in vehicles fitted with electronic engine management systems and ABS (Anti–skid Braking Systems). Under certain fault conditions, emitted RF energy can affect their operation. If necessary, consult the vehicle dealer/manufacturer to determine the immunity of vehicle electronic systems to RF energy.

2. Phones must not be operated in areas likely to contain potentially explosive atmospheres, e.g. petrol stations (service stations), blasting areas etc.

3. Operation of any radio transmitting equipment, including cellular telephones, may interfere with the functionality of inadequately protected medical devices. Consult a physician or the manufacturer of the medical device if you have any questions. Other electronic equipment may also be subject to interference.

1.2 Cautions

1. Repair services and calibrations must be undertaken by qualified personnel only.

2. Use only approved components as specified in the parts list.

3. Ensure all components, modules screws and insulators are correctly re-fitted after servicing and alignment.

4. Electrostatic discharge is the main cause of damage to sensitive components of electronic products. Service centers must operate in accordance with OPPO's requirements for ESD protection. Ensure all work is carried out at an ESD workstation and that an ESD wristband is worn.

2. General Repair Information

1. Ensure all work is carried out at an ESD workstation and that an ESD wristband is worn before starting maintenance.

2. Wear ESD gloves to avoid oil stains and fingerprints.

3. Use protective films to protect the display, camera, camera lens to prevent dust and scratches.

4. Use a clean cloth, ESD brush and alcohol (Concentration above 95%) for appearance cleaning. Do not use other items (such as an eraser) for cleaning to prevent the protective layer from being scratched, which may lead to oxidation and corrosion.

5. Faulty welded mechanical parts (except shield cover and shield frame parts) can only be replaced and not repairable.

6. Use accessories provided by OPPO.

7. Check the contacts or solder joints of devices that may cause simple faults (e.g. soldered interfaces or switches). Re-weld if necessary (only for service centers where lead-free soldering is available) and clean the flux remaining after soldering.

8. Use the equipment provided by OPPO to test the phone (e.g. When the charging function of the device is abnormal, do use the original OPPO adapter to test to ensure the accuracy of the test results).

9. Fill in the fault phenomenon code and fault reason code according to the actual situation, and accurately enter the replacement material code in the CRM system.

10. For more information about products,

Mainland China please visit:

https://kbase.oppoer.me/kbaseui-oppo/login.html

Regions except for Mainland China please visit:

ftp://ftpex.oppo.com:919 or https://oppo-knowledge.custhelp.com

3. Phone Structure Diagram

3.1 Product Appearance

As shown in Figure 3-1-1

Figure 3-1-1

3.2 Phone Structure Explosive View

As shown in Figure 3-2-1

3.3 Explosive Map BOM

| NO | Material name | Picture | Sub-Category | QTY | Unit |
| --- | --- | --- | --- | --- | --- |
| 1 | Screen |  | (12) TP+Display | 1 | PCS |
|  |  |  | (11) Top cover | 1 | PCS |
|  |  |  | (8) Receiver | 1 | PCS |
|  |  |  | (19) Z axis linear motor | 1 | PCS |
|  |  |  | (9) Proximity sensor | 1 | PCS |
|  |  |  | Screen guard | 1 | PCS |
| 2 | Back cover |  | (2) Back cover | 1 | PCS |
|  |  |  | (1) Camera lens | 2 | PCS |
| 3 | Antenna Board |  | (16) Antenna Board | 1 | PCS |
|  |  |  | Waterproof PET sheet | 1 | PCS |
| 4 | Mainboard |  | (12) Mainboard | 1 | PCS |
|  |  |  | Mainboard waterproof PET sheet | 1 | PCS |
|  |  |  | Test point waterproof PET | 3 | PCS |
| 5 | (3) Rear Main Camera |  | Copper foil for rear camera(main) | 1 | PCS |
|  |  |  | Main camera IMX766 50M 15.3×15.3×7.46 6P BTB C0QS01 | 1 | PCS |
|  |  |  | Rear wide-angle camera IMX481 16M 8×8×5.775 5P BTB C0EF19 | 1 | PCS |
|  |  |  | Rear camera prefixed foam | 1 | PCS |
|  |  |  | Rear telephoto camera S5K3M5SX04 13M 8.8×7.7×6.46 5P BTB C0DA57 | 1 | PCS |
|  |  |  | Rear macro camera OV02B10 2M 5.7*6.0*3.171 3P+IR BTB AF240C | 1 | PCS |
| 6 | USB-FPC |  | (6) FPC patch semi-finished product UBD356-SA-patch 20135 | 1 | PCS |
| 7 | L-board |  | (7) FPC patch semi-finished product UBD356-SB-patch 20135 | 17+4 | PCS |
| 8 | A2-board |  | (14) PCB semi-finished A2BD356-SA 20135 | 1 | PCS |
| 9 | Speaker |  | (15) Speaker-BOX 1.14W 6Ω 43.25*19*4.5 A | 1 | PCS |
| 10 | Front camera |  | (10) Front camera IMX615 32M 7.755*7.365*4.92 5P BTB C0HF36 | 1 | PCS |
| 11 | RF cable |  | White 50Ω 128.5mm 818021691 | 1 | PCS |
|  |  |  | Black 50Ω 190mm 818021690 | 1 | PCS |
| 12 | Battery |  | (21) @ 2200mAh*2 7.74V 3.0C BLP825 503284 NA I934 | 1 | PCS |
| 13 | SIM card slot |  | (20) SIM card slot | 1 | PCS |
| 14 | Finger print sensor |  | (22) GW9578 5.8*4.4*2.85mm BTB BD353 A102 UD black | 1 | PCS |
| 15 | Vibration motor |  | (6) φ8*3.2mm 1.8V 23Ω 235Hz | 1 | PCS |
| 16 | Camera lens |  | (1) camera lens | 1 | PCS |
| 17 | Antenna board bracket |  | (13) BD355 black with foam | 1 | PCS |
| 18 | Mainboard bracket |  | (4) Bracket with flash and color temperature | 1 | PCS |
| 19 | Receiver |  | (8)1W 7Ω 10*12*2.8 AAC |  |  |

Note: This table is used only for the name of the annotation. It is not suitable for the preparation of materials. Please refer to the “Price list of Commonly-used Materials and Accessories” for the preparation.

# 4. Disassembly Steps

## 4.1 Disassembly tools required as shown below:

## 4.2 Preparation

The phone must be shut down with the power option before disassembling. Way to Shut Down: Press the power button then slide to power off.( The forced shutdown method is long press the volume + button and power button for about 8 seconds until the display is off)

## 4.3 Precautions for disassembly

| Steps | Details | Picture | Cautions |
| --- | --- | --- | --- |
| 1 | Remove the back cover |  | The position in the picture cannot be cut with a plastic disassembly sheet to avoid damaging the grounding pin of the decorative part. |
|  |  |  | If other positions need to be cut with a disassembly sheet, not to use alcohol, which may cause paint peeling of the back cover. |
| 2 | Remove the L-board |  | When removing the L board, pay attention to the bifurcation position of the two ends. You need to pull them up at the same time to remove it to avoid tearing the FPC. |
| 3 | Remove the battery |  | If the double-sided tape is torn off when removing the battery easy-tore paper, we need to re-attach a new double-sided tape when installing the battery. |
| 4 | Remove the antenna board |  | After removing the antenna board, we need to push the SIM bracket waterproof Needle bar back to its original position to prevent loss. |
| 5 | Remove thermal gel |  | After removing the mainboard, we need to remove the old thermal gel. (the old thermal conductivity is poor) |
| 6 | All parts of the phone |  |  |

5. Assembly Steps

5.1 Materials are required to be replaced after disassembly

| NO | Material code | Material name | Picture | QTY | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | 4884508 | Front camera conductive copper |  | 1 | Applicable to mobile phones in regions except for Chinese mainland market |
|  | 4884396 |  |  | 1 | Only applicable to mobile phones in the Chinese mainland market |
| 2 | 5181951 5181952 | Battery pull handle |  | 1 | must be replaced new one if removed battery |
| 3 | 4885895 | Battery easy-tore paper |  | 1 | must be replaced new one if removed battery |
| 4 | 4885915 | Fingerprint module double-sided adhesive |  | 1 | must be replaced new one if removed battery fingerprint sensor |
| 5 | 4884459 | Speaker/Receiver double-sided adhesive |  | 1 | must be replaced new one if removed Speaker or Receiver |
| 6 | 4881285 | Front camera fixed double-sided adhesive |  |  | must be replaced new one if removed front camera |
| 7 | 4884464 4886503 | Battery cover double side adhesive |  |  | 4886503 is applicable to Artist Limited Edition version |

5.2 Precautions for assembly

| NO | Details | Picture | Cautions |
| --- | --- | --- | --- |
| 1 | Assemble the receiver speaker |  | In order to achieve the stereo sound effect, the double-sided tape used is strong glue. Before installing the new receiver/speaker, the old double-sided tape must be removed and the new double-sided tape must be pasted to avoid noise problems. |
| 2 | Add thermal gel |  | Before installing the motherboard, we need to add a new thermal gel. If we forget to add it, it may cause serious heating of the phone. |
| 3 | Replace the copper foil of the front camera |  | When changing the motherboard or changing the camera, the copper foil may be deformed and damaged. In order to ensure good grounding and heat dissipation performance, new materials need to be replaced |
| 4 | Replace the front camera fixed double-sided adhesive |  | This material needs to be replaced when replacing the motherboard, display, and front camera. The old double-sided tape needs to be cleaned up before replacement. |
| 5 | Replace the fingerprint module double-sided adhesive |  | When replacing the display screen, U board, and fingerprint module, the double-sided tape of the fingerprint module needs to be replaced to avoid fingerprint recognition problems after installation. |
| 6 | Install the RF cable |  | Pay attention to the position of the black and white RF cable on the mainboard |

6. Calibration

6.1 Leak Calibration

| Operation step | 1. Make sure there is no material covering the phone, then press the infrared access calibration button. | 2. Put the upper part of the phone into the airbag and click ”Leak Calibration” | 3. After the calibration result is Pass, the calibration will automatically jump to the fingerprint calibration item after the calibration is completed. |
| --- | --- | --- | --- |
| Picture |  |  |  |

6.2 Fingerprint optical calibration

| Operation step | 1. Click "START" to start calibration after it jumps into the fingerprint optical calibration, | 2. According to the prompt "please place the yellow rubber ", put the yellow rubber on the fingerprint spot area (the groove is facing down), and click "NEXT" until it prompts " please place the black rubber". | 3.According to the prompt "please place the black rubber", put the black rubber on the fingerprint spot area (the groove is facing down), click "NEXT" until the "please place the yellow stripe rubber " prompts to remove the black rubber ; |
| --- | --- | --- | --- |
| Picture |  |  |  |
| Operation step | 4. According to the prompt “Please place the yellow- striped rubber”, put the yellow stripe rubber (striped down) in the spot area, click “NEXT”; | Note: The stripe direction of the yellow stripe rubber should follow the long side of the phone. | Use the new fingerprint calibration tool |
| Picture |  |  |  |

6.3 After-sale Triple Camera Calibration

6.3.1 Calibration Scenario

1. When the portrait mode is poorly blurred, the after-sales service center needs to calibrate the phone.

2. All devices must be calibrated after the phone has disassembled, or the portrait mode will be poorly blurred.

63.2 Calibration Tool

After-sales calibration Chart,

As shown in Figure 6.2.2-1:

Figure 6.2.2-1

6.3.3 Calibration Method

Please align the yellow box with the edge of the chart, and then take a Photo. Then, do not operate the phone when the phone enters the computing interface until it displays “Pass” with the calibration completed.

| Calibration items | Calibration interface | Calibration calculation process | Calibration result |
| --- | --- | --- | --- |
| Main + wide |  |  |  |
| Main + tele |  |  |  |

7. Function test

7.1 Test Path

Dialing Interface>Enter*#899#>Enter automatic test； The "Confirm" dialog box will pop up after the test complete, if there is no problem choose “Pass”, otherwise choose “Fail”.

7.2 Test Requirements

| No | Testing item | Testing requirements |
| --- | --- | --- |
| 1 | LCD test | Observe the red, green, blue, white, black, gray, grayscale and color interface successively; If there are black spots, bright spots and other anomalies, the function is sound; |
| 2 | Vibration test | Get ear close to check the vibration from the motor for 3 seconds. If the motor vibrates normally(No noises or weak vibration), the function is sound; |
| 3 | Touch screen auto test | If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 4 | Front camera preview | Align the front camera with a white surface and a black surface. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, and there is no bright spot on the screen after aligning the black surface, the camera is sound; |
| 5 | Rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is sound; |
| 6 | Second rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is sound; |
| 7 | Third rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is sound; |
| 8 | Fourth rear camera preview | Align the rear camera with a white surface, a black surface, and a surface with text. If there is no black dot, black line or blurred screen on the screen after aligning the white surface, there is no bright spot on the screen after aligning the black surface, white point or blurred screen, the camera is sound; |
| 9 | Echo test | Testing Main MIC: Blow the air to the MIC hole on the bottom of the phone. If the receiver (earphone) sounds, the function of Main MIC is sound; Testing Sub MIC: Blow the air to the sub MIC hole on the top of the phone. If the speaker sounds, the function of Main MIC is sound; |
| 10 | Flash-light test | Open the flashlight. If it can be opened normally with color deviations, the function is sound; |
| 11 | Supper VOOC Charger | Use the original OPPO Supper VOOC adapter and USB cable to charge. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 12 | Headset in-out plug test | Plug in and out the earphone with the OPPO earphone. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 13 | Sensor self-test and calibration | When the phone enters into the Sensor self-calibration interface, the phone must be placed on the desk flatly. Then click the first calibration item. After the test is passed, it will automatically jump to the next test item |
| 14 | Gyroscope | Take the mobile phone to draw the "8" word graphic. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 15 | M-Sensor | Shake the phone from the left to the right side. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 16 | Proximity test | Use the palm to cover the light sensor hole, and the screen will turn green from black. If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 17 | Keypad test | Press the power key and volume key one by one. If keys can be pressed normally and the interface jumps to the next test, the function is sound; |
| 18 | Finger print auto test | If the interface shows “PASS” and jumps to the next test, the function is sound; |
| 19 | Media test | Click the “20-4K（-3db）signal” to test the sound of the media function for 5s. If there is no silence, weak sound, noises, and breaking sound, the interface shows “PASS” and jumps to the next test, the function is sound; |
| 20 | Call test | If the phone has been installed with the operator's SIM card, dial the corresponding operator's phone number. If no card is installed, call the 112 to test (Calling the Operator is recommended). If the call can be performed normally and the receiver is without noises or weak sounds, the function is sound; |
| 21 | NFC test | Close the phone of the rear camera area to the NFC card reader. If the signal light of the card reader changes from red to green with “Beep” sound, the interface shows “PASS” and jumps to the next test, the function is sound. |

After the above 21 items are tested, they will prompt to the test result interface. If the test PASS, project will show “√”; if the test FAIL, project will show “×” as shown below.

8. Cautions

1. When removing the back cover, if we need to use the disassembling sheet to cut open, do not add alcohol in the gap of the back cover, which may cause the back cover paint peeling；

2. When disassembling the phone, please use a protective film to protect the components to prevent dust from contacting；

3. It is required to clean the original thermal gel when replacing the screen or mainboard, and apply new thermal gel according to the above installation steps.

Disassembly and Assembly Video download link：https://fileload.coloros.com/487515Reno5%20Pro%205G%20%26%20Find%20X3%20Neo%20%20Disassembly%20and%20Assembly%20Teaching%20Video.mp4

| Version | Date | Drafted | Verified | Approved | Modified content |
| --- | --- | --- | --- | --- | --- |
| V1.0 | 2021-02-19 | HuYun | Li BinBin | ChenWei | 1st Version |